- Ball Mill

The ball mill is a key equipment to grind the crushed materials, and the ball mill is widely used in powder-making production line including cement, silicate, new-type building material, refractory material, fertilizer, ore dressing of ferrous metal and non-ferrous metal, glass ceramics, etc, and the ball mill can grind various ores and other materials with dry type and wet type. There are two kinds of ball mill, grate type and overfall type due to different ways of discharging material.

- Working Principle:

The ball mill is a horizontal rotating device transmitted by the outer gear. The materials are transferred to the grinding chamber through the quill shaft uniformly. There are ladder liner and ripple liner and different specifications of steel balls in the chamber. The centrifugal force caused by rotation of barrel brings the steel balls to a certain height and impact and grind the materials. The ground materials are discharged through the discharging board thus the grinding process is finished.

- Technical Data:

| Model | Speed of bucket (r/min) | Weight of ball (t) | Size of feed opening (mm) | size of outputting feed (mm) | Production (t/h) | Power (kW) | Weight (t) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ф900×1800 | 38 | 1.5 | ≤20 | 0.075-0.89 | 0.65-2 | 18.5 | 3.6 |
| Ф900×3000 | 38 | 2.7 | ≤20 | 0.075-0.89 | 1.1-3.5 | 22 | 4.6 |
| Ф1200×2400 | 32 | 3.8 | ≤25 | 0.075-0.6 | 1.5-4.8 | 45 | 12.5 |
| Ф1200×3000 | 32 | 5 | ≤25 | 0.075-0.4 | 1.6-5 | 45 | 12.8 |
| Ф1200×4500 | 32 | 7 | ≤25 | 0.075-0.4 | 1.6-5.8 | 55 | 13.8 |
| Ф1500×3000 | 27 | 8 | ≤25 | 0.075-0.4 | 2-5 | 90 | 17 |
| Ф1500×4500 | 27 | 14 | ≤25 | 0.075-0.4 | 3-6 | 110 | 21 |
| Ф1500×5700 | 27 | 15 | ≤25 | 0.075-0.4 | 3.5-6 | 132 | 24.7 |
| Ф1830×3000 | 24 | 11 | ≤25 | 0.075-0.4 | 4-10 | 180 | 28 |
| Ф1830×6400 | 24 | 23 | ≤25 | 0.075-0.4 | 6.5-15 | 210 | 34 |
| Ф1830×7000 | 24 | 25 | ≤25 | 0.075-0.4 | 7.5-17 | 245 | 36 |
| Ф2200×5500 | 21 | 30 | ≤25 | 0.075-0.4 | 10-22 | 370 | 48.5 |
| Ф2200×6500 | 21 | 30 | ≤25 | 0.075-0.4 | 14-26 | 280 | 52.8 |
| Ф2200×7500 | 21 | 33 | ≤25 | 0.075-0.4 | 16-29 | 380 | 56 |
| Φ2200×9500 | 21 | 38 | ≤25 | 0.074-0.4 | 18-35 | 475 | 62 |

Notice: Any change of Ball Mill technical data shall not be advised additionally.

Contact Us:

Address:		No. 15, Dingxiangli Road, National HI-TECH Industry Development Zone, Zhengzhou, China.

Postcode: 	450001

Tel: 		0086-371-67992899	67371699 	67999318 	67996989

Fax: 		0086-371-67992699

Email: 		Break-day@break-day.com

MSN Online:	MiningMachine@hotmail.com

MSG Online: Summer_8001@yahoo.com

Skype:		miningmachinery